Table 1. Healthcare, Pharmaceutical and Food Work Environments

|  | Sampling technique (microbiological parameter) | Benchmark Guidance Value | Microbiological indicator | References |
| --- | --- | --- | --- | --- |
| Hospitals | Dipslide (ACC 30°C/48h) | ≤ 2,5 CFU/cm2 | Staphylococcus aureus; MRSA; MSSA | [64] |
|  | Contact plates (ACC 37°C/48h) | ≤ 2,5 CFU/cm2 |  | [65] |
|  | ACC | < 5 CFU/cm2 | Staphylococcus aureus; MRSA; Clostridium difficile, Salmonella; Aspergillus Vancomycin-resistant enterococci | [66] |
|  | ATP-bioluminescence | < 100 RLU |  | [65, 64] |
|  | ATP-bioluminescence | < 250 RLU | Staphylococcus aureus; MRSA | [56] |
|  | Contact plates (TBC) | From ≤ 5 CFU/plate (operating room and other critical environment) to ≤ 50 CFU/plate (ICU-Intensive Care Unit, neonatology) | Staphylococcus aureus; Aspergillus spp.; Pseudomonas spp.; Enterobacteria | [31] |
|  | Contact plates (TBC 37°C) | Low risk area (office) < 5 CFU/cm2 Medium risk area (waiting room, lifts, counselling etc.) < 2 CFU/cm2 High and very high risk area (ICU, operating room, neonatology, emergency, etc.) < 0,2 CFU/cm2 | Aspergillus fumigatus | [37] |
|  | Swabs (ACC 37°C/48h) | < 2,5 CFU/cm2 | MRSA | [56] |
|  | Swabs (ACC 48h) | < 2,5 CFU/cm2 | S. aureus; E. coli; P. aeruginosa; A. baumannii | [67] |
| Ambulances | Swabs (TBC 37°C/48h) | < 5 CFU/cm2 |  | [68] |
| Surgery practices | Swabs (TBC 37°C/72h; Total Fungal Count: TFC 28°C/72h) | 0 CFU/cm2 (acceptable) 1-5 CFU/cm2 (doubtful) > 5 CFU/cm2 (not acceptable) | Enterobacteria; P. aeruginosa | [69] |
| Dental practices | Contact plates (TBC 36°C/48h) | ≤ 0,64 CFU/cm2 (acceptable) 1,48 CFU/cm2 (alert) |  | [70] |
|  | Swabs (TBC 36°C/48h) | ≤ 1 CFU/cm2 |  | [71] |
| Pharmaceutical Clean rooms | RODAC plates (TBC 30-35°C/48h, than 20-25°C/72h ) | Class A1: < 1 CFU/plate (walls and benches) Class B1 and B2: < 5 CFU/plate (walls); < 10 CFU/plate (benches) Class C: < 25 CFU/plate (walls); < 50 CFU/plate (floor) Class D1 and D2: < 50 CFU/plate (floor and walls) | Staphylococcus spp.; Micrococcus spp.; Bacillus spp.; Candida spp. | [36] |
|  | Swabs | Class A2: < 1 CFU/swab |  |  |
|  | Contact plates | Surfaces: from < 1 CFU/plate (sterility level A) to < 50 CFU/plate (sterility level D) Gloves: from < 1 CFU/glove (zone A) to < 5 CFU/glove (zone B) |  | [30] |
| Pharmaceuticals | Contact plates (TBC 37°C/24h, than RT/48h) | From < 0,2 CFU/cm2 (clean rooms) to < 5 CFU/cm2 (offices, stairs, etc.) | Staphylococcus spp.; Micrococcus spp.; Bacillus spp.; Candida spp. | [37] |
| Food preparation areas (surfaces and tools) | Contact plates (25,8 cm2 20-25°C/48h) | < 150 CFU/plate | Pseudomonas fluorescens; Escherichia coli; S. aureus | [72] |
|  | Contact plates (TBC) | ≤ 10 CFU/cm2 (after sanitization) | Salmonella spp.; Listeria monocytogenes; Total coliforms ≤ 1 CFU/cm2 | [73, 74] |
|  | Contact plates (TBC) | Satisfactory sanitization: TBC ≤ 10 CFU/cm2 | Total coliforms ≤ 1 CFU/cm2 Absence Salmonella and Listeria monocytogenes | [75] |
|  | Contact plates (TBC 30°C/48h; TFC 25°C/120h) | 0 ≤ CFU/plate ≤ 2 (very good) 3 ≤ CFU/plate ≤ 9 (good) 10 ≤ CFU/plate ≤ 29 (satisfactory) | Coliforms; S. aureus | [36, 76] |
|  | Swabs (TBC) | 0 ≤ CFU/100 cm2 ≤ 8 (very good) 12 ≤ CFU/100 cm2 ≤ 36 (good) 37 ≤ CFU/100 cm2 ≤ 116 (satisfactory) | Coliforms; S. aureus | [36, 76] |
| Catering | Contact plates (TBC) | ≤ 4 CFU/cm2 | Total microorganisms, coliforms, S. aureus, E. coli, Salmonella spp. and Listeria monocytogenes | [77] |
| Restaurants | ATP-bioluminescence | 50 RLU | Escherichia coli | [78] |
|  | Swabs (TBC 30°C/24-48h) | 80 CFU/cm2 |  |  |
| Butcheries and supermarkets | Contact plates (TBC) | ≤ 4 CFU/cm2 | Total coliforms < 1 CFU/cm2; E. coli | [79] |
| Area microbiologically checked (cooked dishes) | Contact plates (TBC 37°C/24h and room temperature/48h) | Area 1 < 5 CFU/cm2 Area 2 < 2 CFU/cm2 Area 3 < 0,2 CFU/cm2 Area 4 < 0,2 CFU/cm2 | Coliforms and E.coli | [37] |

Legends - TBC: Total Bacterial Count; ACC: Aerobic Colony Count; RLU: Relative Light Units